|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Band saw |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | - Tilting workpieces can tear the band saw blade and cause serious injury. - Contact with the band saw blade causes cuts.. - In case of strong noise (from 80 dB (A)) there is a risk of hearing damage. - The running band saw blade can grasp and pull in the work clothes. - Chips and sharp-edged components may cause cuts. - Danger of accident due to damaged band saw blades (for example blunt, torn). - There is a risk of skin diseases when working with cooling lubricants. |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | - The operating instructions of the manufacturer must be observed! - Avoid tilting the workpiece when the workpiece is fed to the band saw blade. - Dress band saw blade to the part required for cutting. - Contact with the band saw blade must be avoided. - Use personal protective equipment (including skin protection). - Wear tight-fitting work clothes. - Switch off the machine even during short interruptions and cover the saw blade behind. - Do not remove splinters, chips and waste by hand, but with suitable tools. - Do not use damaged band saw blades. - It is forbidden to wear protective gloves (danger of getting into the rotating blade). - To wear ear protection. - If there is a risk of eye injuries, the goggles must be worn. - Switch off the machine even during short interruptions and cover the saw blade behind. |  |  |  | - pE |
| ## Faults and dangers |  |  |  |  |  |  |
|  |  | • If work equipment fails, discontinue work and inform the responsible teacher. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | - Attract first responders - Emergency call: 144 - Report accident to responsible |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - Maintenance (maintenance, repair) should only be carried out by qualified and authorized persons. - After maintenance, the protective equipment must be checked. - Observe the operating instructions of the manufacturer during maintenance. - Regular checks (e.g., electrical, mechanical) by qualified persons. |  |  |  |  |  |  |
| ## Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |