|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Compressed gas cylinders |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | - Risk of injury when falling over. - Release the stored pressure energy by mechanical damage u. unintentional opening of the valve. - Explosion of gas bottles in case of excessive heating. - Health and fire hazard due to release of gases due to leaking gas connections. - Uncontrollable reactions of the contents gases due to the ingress of air or oil (oxygen) due to the backflow of gaseous u. liquid foreign substances e.g. from other gas bottles. |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | - Do not carry or roll gas bottles, but transport them with a bottle cart. Only transport with protective cap. - Wear protective shoes and protective gloves - Secure against falling over (chaining). Put on protective cap. - Open the gas cylinder valves slowly to avoid pressure surges. - Close valves at the end of work. - Empty gas cylinders only to a residual pressure (approx. 1.5 bar). - Do not touch the gas cylinders with the flame, do not oil or grease, do not force open. - protection of gas cylinders against fire hazard, e.g. Do not store together with flammable liquids. - Keep away from sources of ignition and heat. |  |  |  | - pE |
| ## Faults and dangers |  |  |  |  |  |  |
|  |  | - In the event of damage to the gas cylinders or the valves: close the valves and inform the workshop management. - In case of damage to the protective equipment or other faults, inform the workshop management. - Have damage repaired by qualified personnel only. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | - Close the gas cylinder. Close valves. - Inform first responders (see alarm plan). - Provide minor injuries immediately. - make an entry in the association book. - In the case of major injuries, consult a general practitioner (see "First Aid") or call the emergency medical service via Tel.144 - Inform the workshop management .. |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - Defects on the gas cylinder / on the fittings must be reported to the workshop management immediately. - Repair only by instructed and instructed persons |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |