|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work - Equipment - Apparatus - Device |  |  |  |  |  |  |
| Dichlormethane |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  |  | Causes skin irritation. (H315) Causes serious eye irritation. (H319) Can probably cause cancer. (H351) May irritate the respiratory system. (H335) May cause drowsiness and dizziness. (H336) Can cause damage to organs through prolonged or repeated exposure. (H373) - Inhalation, ingestion or skin contact may result in adverse health effects. When odor perception, leave workstation! Irritating to eyes, skin. May irritate respiratory system and gastrointestinal tract. Temporarily dizziness, headache, drowsiness and difficulty concentrating possible. May cause liver damage, nerve damage and arrhythmia. At higher concentrations there is danger of suffocation. Carcinogenic effect of dichloromethane is suspected! - Danger due to accumulation of explosive atmosphere near the ground! Ignition by flames or welding sparks possible. In the event of rapid leakage from the container, ignition due to electrostatics is possible. Increased risk of ignition of soaked material (e.g., clothing, cloths). Reacts violently with alkali metals, alkaline earth metals. Unstabilized dichloromethane reacts violently with light metals. Reacts with strong oxidants with vigorous evolution of heat. Reacts with strong alkalis under intense heat. Reacts with vigorous evolution of heat e.g. with nitrogen oxides. An uncontrolled reaction may cause a risk of explosion. Forms flammable gases / vapors with potassium tert-butoxide, liquid oxygen. - Dangerous reactions in the workplace are possible with: - Avoid penetration into soil, water and drains! |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  | - When working in dampenings or in furniture, pull and close the front of the machine. Do not open containers! When pumping and refilling, avoid spraying and castering. Keep substances capable of reacting at a controlled temperature or only give them in a controlled manner. At temperatures above approx. 40 ° C, an overpressure may build up in closed containers. Carefully close closures of containers only after pressure equalization! Do not use pipettes, make solutions using weighting scales. For dosing e.g. Use cooled micro liter syringes and pre-cool the liquid. In a rotary evaporator, remove only under normal pressure. Use tools that prevent or reduce skin contact. - Keep stocks in the laboratory as low as possible, and protect them against flame and heat. Keep away from ignition sources (for example not smoke, no open flames, earth)! Fire-Works (for example, maintenance, repair) only with written permission. - Do not eat, drink, smoke or runny nose. Avoid breathing vapors! Contact with eyes, skin and clothing avoid! After work and before each break, hands and other soiled parts of the body should be thoroughly rinsed. Skin care agent is used! Remove product residues immediately from the skin, gently clean the skin and dry thoroughly. Do not wear arm or hand jewelry. Street clothing kept separate from working clothes! Change soiled and saturated work clothes immediately. Use separate cleaning rags and cleaning cloths for skin and machinery or equipment. - Observe occupational restrictions Stock quantity at the workplace: Eye protection: Protective goggles with side protection. In case of danger splashing: Goggles! Hand protection: Gloves made of: (Please select a suitable protective glove from the Gis Chem data sheet or other information available to you in consideration of the activity and indicate this here.) When wearing protective gloves, cotton gloves should be worn Shoes recommendable! Goggles of PE / EVAL / PE (PE = polyethylene, EVAL = Ethylene-Vinyl-Alcohol -Copolymer) use only as Injection protection. Pay special attention to hand and skin protection - the substance can also enter the body through the skin. Pay attention to wearing times of protective gloves! For long-term use of protective gloves: use special skin protection before. |  |  |  |  | - pE |
| ## Disruptions and Dangers |  |  |  |  |  |  |
|  | Clear and close the danger area, inform supervisors. Always wear protective goggles, hand shoes and, for larger quantities, respiratory protection when cleaning out leaking / spilled product. Suitable respiratory protection: Respiratory mask: Half mask filter class ABEK (as in emergency kit MUI included),! Note operating times. Absorb with an absorbable, non-combustible material (e.g. Diatomaceous earth, sand,) and desorbed! Product does not burn under normal conditions. In the event of fire, replace extinguishing measures with ambient air. In the case of fire, hazardous vapors (for example chlorine, hydrogen chloride, phosgene, carbon monoxide) are formed! Precipitating vapors with spray water, subsequently as fast as possible cleaning. Danger of bursting when heated. Fire fighting from close range only with acid-resistant protective clothing! Observe alarm, escape and rescue plans. Alert fire department. Occupational Physician: Dr. med. Olaf Riccabona 0512/504 27527 0664/5210561 Emergency call: 144 |  |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  | For each first aid measure: pay attention to self-protection, medical treatment. Life-saving immediate measures, such as stable lateral position, cardiopulmonary resuscitation, and shock reduction must be carried out depending on the situation. Cover wounds germ-free. Ensure body peace, protect against heat loss. After eye contact: Immediately wash (with protection of the uninjured eye) extensively (about 10 minutes) with open eyelids with water. After skin contact: Remove uncleaned clothing, also undergarments and shoes, undressed. Rinse skin with plenty of water, if necessary with PEG 400. Inhalation: Move injured person out of the danger zone. Fresh air supply by inhalation of fresh air or respiration. Use ventilation aids (self-protection). Ingestion: Immediate vigorous rinsing of the mouth. Allow water to drip in small swirls. first responders: |  |  |  |  |  |
| Do not pour into spouts or garbage bins! Collect substance / product waste for disposal in: Packaging with residual contents: Collect uncleaned absorbent material and cleaning cloths in: |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |

Tests - maintenance - disposal