|  |  |  | Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Equipment - Apparatus - Device |  |  |  |  |  |  |
| Double - grinders |  |  |  |  |  |  |
| ##### Dangers of environment |  |  |  |  |  |  |
|  | - Workpieces may heat up during processing - danger of burns - Injuries due to abrasive disc breakage or defective abrasive belt - Eye injuries from worn materials - Foot injuries caused by falling workpieces - Hand injuries due to sharp-edged surfaces - Hearing damage due to noise - Machine caster - Collection of clothing and / or hair, gloves - flying sparks, danger of fire - Health hazard due to the formation of dusts (see material properties) |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | - The operating instructions of the manufacturer must be observed! - Wear protective goggles and shoes suitable for the area - Depending on the materials and activities, additional dust mask and ear protection must be worn - The clamping of a suitable grinding wheel / belt may only be carried out by instructed employees. After each new clamping a test run of ~ 20 minutes is to be carried out. - Workpiece supports and protective covers must be adjusted regularly. The distance between the grinding wheel / sanding belt and the workpiece support must not exceed 3 mm, between the grinding wheel and the sanding hood at most 5 mm. If necessary, wear only suitable gloves. - Wear tight fitting clothes - Wear hairnet for long hair - Take fire protection precautions. For example, remove flammable substances / materials |  |  |  | - pE |
| ## Disruptions and dangers |  |  |  |  |  |  |
|  |  | - In case of malfunction or damage to machinery or protective equipment - Switch off the machine and secure against unauthorized restarting - Have damage repaired by qualified personnel only |  |  |  |  |
| ## Accidents and first aid |  |  |  |  |  |  |
|  |  | - Keep Calm - observe self-protection; Injured people recover, switch off the system - Call first responder and, if necessary, emergency doctor. Emergency call: 144 - provide first aid - Inform supervisors - An accident report must be made in the case of accidents and near-misses. |  |  |  |  |
| ## Tests - Maintenance - Disposal |  |  |  |  |  |  |
| - Repair only by authorized and qualified persons - For setup, maintenance and care work, disconnect or secure the machine from the mains - After maintenance, the protective equipment must be checked. - E-check, depending on the model, annually or every four years by a trained electrician (stationary or mobile machine) |  |  |  |  |  |  |
| ## Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |