|  |  |  | ### Operating instructions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |  |
| Drying Ovens |  |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |  |
|  |  | There are hazards due to: - Risk of burns from fingers and hands on hot surfaces. - Danger of crushing the fingers when pushing in the hordes or when closing the door. - Irritation of the respiratory tract due to dust formation, especially with dried material. - Irritation of the respiratory tract due to substances contained in the product, e.g. Solvent. - Possibly. Irritation of the skin in direct skin contact with the dry matter. - Risk of explosion in the presence of flammable solvents - Risk of fire in combustible dry matter due to spontaneous combustion - Slipping hazard due to spilled product. |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |  |
|  | - The employees must be instructed in the handling of the drying oven. - Wear mandatory PPE (basis: risk assessment). - Take care when handling the substances to be dried, if necessary observe BA hazardous substances. - Pick up spilled product immediately and dispose of according to the operating instructions. - Clean and care for hands after contact with the product (see skin protection plan). - Extract exhaust air from the drying oven as far as possible to the outside, if necessary clean it with an exhaust air scrubber. - Always close the drying oven well. - For drying with flammable solvents, use only Ex-protected technical equipment. - Observe Ex protection guidelines. |  |  |  |  | - pE |  |
| ## Faults and dangers |  |  |  |  |  |  |  |
|  | - Do not use the drying oven in case of defective or faulty extraction. - Only rectify faults in the electrical heating by qualified electricians. - Do not use the drying oven in case of defective or faulty extraction. - Only rectify faults in the electrical heating by qualified electricians. |  |  |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |  |
|  | - In case of skin contact with the product to be dried, rinse with plenty of water, consult a doctor if necessary. Injured skin increases the health risk significantly. - Depending on the type of injury, provide first aid as far as possible, call for help if necessary. - Instruct assistants and point out special dangers. - Emergency call: 144 |  |  |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |  |
| - Regularly check drying oven for proper condition (see manufacturer's recommendation). - Dispose of drying cabinets only after sufficient cleaning in accordance with legal regulations (electronic waste). |  |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |  |