|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Edge Bank |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | - Risk of severe cuts due to sharp edged metal sheets - Bruising by moving up and down the bending beam or through the upper beam - Jumping out of improperly clamped workpieces - Clamping clothes when clamping workpieces |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | - The operating instructions of the manufacturer must be observed! - Never override safety devices (for example, to clear jammed workpieces) - Do not exceed the prescribed thicknesses or bending radii - Keep the workplace clean and tidy. Store workpieces to be machined only in the immediately required quantity in the work area - Pay attention to the distance of the hands when setting the workpieces - Stepping out of the swivel range of the bending beam - Wear tight fitting clothes - Scarves, watches, hand and arm jewelery are not allowed - wear gloves (against cuts) - Working with two people only after consultation with the responsible person - Do not put your hands in the danger area |  |  |  | - pE |
| ## Faults and dangers |  |  |  |  |  |  |
|  |  | - Secure the device in the event of damage to the device or its protective equipment - Have damage repaired by qualified personnel only |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | - secure the device - Inform first responders - Provide immediate injury - Emergency call: 1 |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - Repair only by authorized and qualified persons - After maintenance, the protective equipment must be checked. - Clean the machine after work |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |