|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Electric welding |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  |  | - Infrared or ultraviolet radiation (arc), flare of the eyes - Flying sparks, danger of fire, burning of the skin - Electric shock - Welding of containers: Danger due to residues of the ingredients! - Welding fumes: health hazard during welding work on high-alloyed workpieces, metallic coatings, paint coatings, plastic coatings, contamination by oils, greases or residual solvents etc. |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  | - For welding work in areas with a fire and explosion hazard, obtain a welding permit - Remove or cover combustible parts from the environment - Provide fire extinguishers - Check the welding generator, cable, electrode holder and ground terminal for defects before use - Establish good conductivity between earth cable and workpiece - Switch on the suction device before starting work and adjust if necessary - Always place the burner in an insulated position (if possible, remove the electrode residue) - Protect cables against damage, use cables with insulated couplings - Do not clamp the welding wire holder and inert gas welding torch under the arm and place it only on insulating shelves - Position protective gas bottle securely and secure against falling over - Observe the operating instructions "Electrical welding under increased electrical hazard" when welding under increased electrical hazard - When welding containers with flammable liquids, empty container completely and fill with water - Use protective shield or protective shield with welding protection filter, wear sweat-proof gloves and clothing, also for welding assistants - Switch off the welding machine before connecting the welding cables After completing the welding work, ensure a fire station |  |  |  |  | - pE |
| ## Faults and dangers |  |  |  |  |  |  |
|  | - In case of defects, do not use the device and inform the supervisor - If the extraction system fails, discontinue work and inform supervisors |  |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  | - Keep Calm - Involve first responders - Emergency call: 144 - To report an accident - In the event of a power accident: - Disconnect the power and remove the injured from the circuit - In case of respiratory or cardiac arrest initiate resuscitation and alert the emergency physician |  |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - Performing maintenance work only by authorized specialist personnel - Replace damaged insulating jaws and welding wire holder immediately - Change the wire reels only when they are de-energized |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |