|  |  |  | ### Operating instructions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ##### Work - Equipment - Apparatus -Device |  |  |  |  |  |  |  |
| Ethidiumbromid |  |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |  |
|  | Danger of death by inhalation. (H330) Harmful if swallowed. (H302) Can probably cause genetic defects. (H341) Ingestion may cause damage to health. Inhaling small amounts can be life-threatening. Irritating to the respiratory system, eyes, skin. Ethidium bromide can potentially lead to hereditary damage! Danger due to accumulation of explosive atmosphere near the bottom when heated! Formation of explosive dust-air mixtures possible. Ignition of dust-air mixtures by open flames or in mills possible. Reacts with strong oxidants with vigorous evolution of heat. |  |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |  |
|  |  | Keep closed lab coats, sturdy and closed footwear and goggles. Eating, drinking, running cold and smoking as well as applying cosmetics are forbidden at work. Do all work with the dry substance under the fume hood. In case of skin contact (eg in case of open activities with the hazardous substance such as weighing or preparing solutions) wear nitrile protective gloves. Protective gloves made of latex are only suitable for short-term protection (ethidium bromide diffuses through). Handle solutions and stained gels with gloves only. Use as ready as possible solutions. If there is a risk of inhalation of aerosols or fine dusts, additionally use FFP3 particulate filter mask. Pay attention to the greatest cleanliness in the workplace, contamination must be eliminated immediately. Keep container tightly closed. Used quantities should be kept cool and protected from light (eg in the fridge). Storage must be done only under lock in the chemical cabinet no. ..... in the room ...... Keep away from oxidizing agents and protect from strong heat. Failure to comply can result in the formation of hazardous gases (NOx, Br2, HBr). |  |  |  | - pE |  |
| ## Disruptions and Dangers |  |  |  |  |  |  |  |
|  |  | Warn endangered persons, if necessary, evacuate and close off the endangered area. The access of unauthorized persons is to be prevented. Contaminations on objects or on the skin can be easily visualized under the UV hand lamp (4-6 watts) (fluoresce pink-orange). Contaminations: When remedying the dangerous condition, attention must be paid to self-protection. At least protective goggles, protective gloves made of nitrile and half-mask filter class ABEK (as included in the emergency kit MUI) must be worn. Absorb solid substances with water-moist pulp. Take up solutions with liquid binder diatomaceous earth (eg Chemizorb). Then rinse used cleaning equipment and wetted surfaces with plenty of water. Fight fires with the nearest available fire extinguisher. In case of personal fires, the emergency shower is also suitable. For larger fires alert the fire department. Occupational Physician: Dr. med. Olaf Riccabona, Tel: 0512/504 27527, 0664/ 5210561 Rescue: 144 |  |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |  |
|  |  | Take off contaminated or soaked clothing (including underwear) and personal protective equipment immediately and use only after thorough cleaning. Inhalation: Move affected person to fresh air. If necessary, administer respiratory or ventilator breathing. Skin: thoroughly cleanse wet skin with plenty of water and rubbed cream several times. Check cleaning success under UV hand lamp (Caution: consider possibility of eye damage due to UV light). Eyes: Immediately flush wet eyes with water for 10 minutes with the eyelid wide open until medical attention is given. Ingestion: Allow plenty of water to drink with activated charcoal! Induce vomiting! Notify the emergency doctor. In case of intensive contact, consult a doctor or call an ambulance. Contaminated safety data sheet, operating instructions and accident accompanying slip. |  |  |  |  |  |
| ## Tests - Maintenance - Disposal |  |  |  |  |  |  |  |
| Dispose of all residues and contaminated waste in labeled closable containers. Soaked material and uncleaned empty containers should be treated like the ingredients. Disposal takes place on demand, but at the latest before the weekend over the building services as special waste (Mr. / Mrs. ....., Tel: ..... if necessary inform). |  |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |  |