|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work - Equipment - Apparatus -Device |  |  |  |  |  |  |
| Fixing agent for light microscopy |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
| Formaldehyde solution: T, C, Xn, Xi Propionic acid: C Ethanol: F Mixing ratio 90T Ethanol 50%: 5T Formaldehyde: 5T Propionic acid chem. formula and details for the individual chemicals see Safety Data Sheets ##### Protective measures and rules of conduct | formaldehyde Toxic if swallowed, in contact with skin, Inhalation Severe burns to the skin and eyes propionic Severe burns to the skin and eyes Flammable liquid and vapor ethanol Highly flammable vapor and liquid in higher concentrations |  |  |  |  |  |
| Disruptions and Dangers Persönliche Schutzmaßnahmen Bei gefährlichen Arbeiten in Laboratorien ist das ständige Tragen einer Schutzbrille, eines Labormantels, sowie festen, geschlossenen und trittsicheren Schuhwerks Pflicht. |  | General information Preparation of the fixing agent under the trigger. Open storage bottles with formaldehyde and propionic acid only under the hood. Do not inhale vapors. Avoid skin contact Keep liquids away from open flames, hot surfaces and sources of ignition The general rules of conduct for chemical work apply (prohibition of Smoking, drinking, eating) Required protective equipment: safety goggles Gloves (nitrile rubber, blue) Lab coat |  |  |  | - pE |
| Use and storage: Fixing mixture in small quantities (1L) can be stored in the laboratory The storage bottles for formaldehyde and propionic acid must be stored in a ventilated chemical cabinet. Production of the fixative and fixation of plant material For the production of 1L FPA50 (trigger, gloves, goggles, lab coat) - Fill 800 ml of ethanol 50% (EtOH) in Schott bottle - Add 50 ml of formaldehyde solution and then 50 ml of propionic acid with a graduated cylinder. - Add the remaining 100 ml EtOH 50% with the same graduated cylinder - Rinse cylinder well with well water first, then with A.dest. Transfer plant material to sample container (type depends on the sample size: Eppendorf tubes - scintillation vials - 4-edge containers ...) and cover well with fixative solution (Gloves, goggles, lab coat). For an optimal fixation effect, the container should only be filled loosely with plant material. Leave plant material in the fixation medium until further processing, at least 24 h. Store specimens in a well ventilated, cool room. Transfer fixed plant material to petri dish containing 70% ethanol for further analysis |  |  |  |  |  |  |
| ## Disruptions and Dangers |  |  |  |  |  |  |
|  |  | Suitable extinguishing agents Water, carbon dioxide, foam, dry powder, Unsuitable extinguishing media: none For formaldehyde and propionic acid, the formation of hazardous combustion gases or Vapors possible; Vapors are heavier than air. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | For formaldehyde and propionic acid After inhalation: fresh air, consult a doctor. After eye contact: Remove contact lenses, rinse thoroughly with plenty of water for at least 15 minutes, consult an ophthalmologist immediately After skin contact: rinse with plenty of water, remove contaminated clothing Ingestion: Rinse mouth with water, drink water immediately (max 2 glasses), DO NOT induce vomiting, do not attempt neutralization, seek medical advice immediately Mehr anzeigen Weniger anzeigen |  |  |  |  |
| ## Tests - Maintenance - Disposal |  |  |  |  |  |  |
| Do not empty used fixative into pour; marked with "FPA50-used" Collect canisters and dispose of chemicals. |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |