|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work - Equipment - Apparatus -Device |  |  |  |  |  |  |
| Formamid |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | Flammable liquid and vapor. (H226) Harmful in contact with skin or if inhaled. (H312 + H332) Causes serious eye irritation. (H319) May damage the unborn child. (H360D) Inhalation, ingestion or absorption through the skin may result in adverse health effects. When odor perception leave workstation! Irritating to eyes. May irritate respiratory system and skin. With frequent skin contact skin inflammations are possible. Prolonged nausea, vomiting, alcohol intolerance, gastrointestinal complaints and seizure-like erythema possible. Alcohol can aggravate health problems. May cause liver damage and damage to the digestive tract. DMF can damage the baby around the womb! Danger due to accumulation of explosive atmosphere near the ground during spraying or heating! Increased risk of ignition of soaked material (e.g., clothing, cloths). Reacts with strong oxidizing agents and strong reducing agents with vigorous evolution of heat. Reacts with vigorous evolution of heat e.g. with carbon tetrachloride or hexachlorocyclohexane in the presence of traces of iron. Reacts with vigorous evolution of heat e.g. with chlorine. An uncontrolled reaction may cause a risk of explosion. Forms combustible gases / vapors with chromium trioxide. Dangerous reactions in the workplace are possible with: Avoid entry to the ground, waterways and sewer |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | In the case of vapors or mists, switch on the suction and work in its area of ​​effect. Do not leave the containers open! Avoid splashing and overrun when filling and decanting. Keep reactive substances away or only add in a controlled manner. Allow workpieces, tools and plant parts to evaporate in a separate drying area after cleaning in cleaning facilities. Thoroughly clean the cleaning device before using a new solvent. Use tools that prevent or reduce skin contact. Spray or avoid heating, otherwise fire and explosion hazard. Do not eat, drink, smoke or sniff. Avoid contact with eyes, skin and clothing! Avoid breathing vapors! Wash hands and other contaminated parts of the body thoroughly after the end of work and before each break. Use skin care products! Remove product residues from the skin immediately, clean the skin and dry carefully. Do not wear arm or hand jewelry. Store street clothing separately from work clothing! Change contaminated and soaked work clothes immediately. Use separate cleaning rags and cleaning cloths for skin and machines or devices. Observe employment restrictions! Amount of stock at work: Eye protection: For surveillance activities: glasses with side protection! If there is a risk of splashing: basket glasses! Hand protection: gloves (Please select a suitable protective glove from the GisChem data sheet or other information available to you, taking into account the activity, and indicate it here.) Cotton protective gloves are recommended when wearing protective gloves! Pay special attention to hand and skin protection - the substance can also get into the body through the skin. Note the wearing times of protective gloves! If protective gloves are worn for a long time: use special skin protection agents before work. |  |  |  | - pE |
| ## Disruptions and Dangers |  |  |  |  |  |  |
|  |  | Clear and close the danger area, inform supervisors. Always wear protective goggles, gloves as well as larger quantities of respiratory protection when cleaning out leaking / spilled product. Respiratory mask: Half mask filter class ABEK (as included in emergency kit MUI). Absorb with absorbent non-combustible material (such as silica, sand) and discard! Product is flammable. Genesis: Use portable fire extinguisher. Not to be used: full jet water! Danger of bursting and explosion when heated! Fire releases hazardous fumes (such as amines, nitrogen oxides, formaldehyde, carbon dioxide)! In case of fire in the area cool container with water spray! Observe alarm, escape and rescue plans. Alert fire department. Occupational Physician: Dr. med. Olaf Riccabona 0512/504 27527 Mobile: 0664/5210561 Emergency call: 144 |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | For each first aid measure: pay attention to self-protection, medical treatment. Life-saving immediate measures, such as stable lateral position, cardiopulmonary resuscitation, and shock reduction must be carried out depending on the situation. Cover wounds germ-free. Ensure body peace, protect against heat loss. After eye contact: Immediately wash (with protection of the uninjured eye) extensively (about 10 minutes) with open eyelids with water. After skin contact: Remove uncleaned clothing, also undergarments and shoes, undressed. Rinse skin with plenty of water, if necessary with PEG 400. Inhalation: Move injured person out of the danger zone. Fresh air supply by inhalation of fresh air or respiration. Use ventilation aids (self-protection). Ingestion: Immediate vigorous rinsing of the mouth. Allow water to drip in small swirls. first responders: |  |  |  |  |
| ## Tests - Maintenance - Disposal |  |  |  |  |  |  |
| Soak saturated cleaning cloths only in special resistant containers that are tightly sealed. Do not pour into a spout or dustbin! Collect substance / product waste for disposal in: Packaging with residual contents: Collect contaminated absorbent material and cloths in: |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |