|  |  |  | Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Gas welding |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | Fire and explosion hazard - inhalation of welding fumes - Dangers to eyes and skin due to UV rays - Danger due to hot surfaces |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | - Check the device and hoses for defects before use (also valid for the inspection labels). - Switch on the suction device before starting work and adjust it if necessary - Protect hoses from damage and prevent standing gas cylinders from falling over - Wear personal protective equipment and ensure a safe condition: - Apply skin protection according to skin protection plan - Wear welding gloves with cuffs (without metal rivets), - Wear protective goggles with a matched protective filter - Wear protective sleeves, gaiters, aprons made of flame retardant material or a welder protective suit. Wear appropriate safety shoes in the area - Make sure that welding assistants wear personal protective equipment - Take anti-glare measures for others (eg close the curtain) - Close taps before coupling the welding hoses and at the end of the work. - Only use welding and soldering equipment after instruction - Only transport gas bottles with the protective cap screwed on - When starting up, first open the oxygen valve slightly. - Secure or remove combustible materials - When finishing work, first close the burner valve - Do not bring oxygen bottles and their fittings into contact with oil or grease - At the end, bend the welding rods due to risk of injury. - Pay attention to further fire and explosion hazards Mehr anzeigenWeniger anzeigen |  |  |  | - pE |
| ## Faults and dangers |  |  |  |  |  |  |
|  |  | - Defects on the gas bottles and hoses must be reported immediately - In case of fire, carry out a fire test |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | - The (first responder informing) - Provide immediate injury - Emergency call: 144 |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - Only repaired by instructed and instructed persons - Clean workplace after work - Regular inspection of gas cylinders and hoses |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |