|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Grinding machine |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | - Cutting injuries on the workpiece - Abrasive damage to abrasive belt and grinding wheel - Fast rotating grinding wheel / belt, trailing grinding wheel / belt - Breakage of the grinding wheel, tearing of the grinding belt - Injuries due to flying workpiece or tool parts - Flying sparks and fire - Noise development and dust generation - Risk of entanglement - Risk of burns on heated workpiecesMehr anzeigen - Weniger anzeigen |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | - Operation of the machine only after instruction. - Wear tight-fitting work clothing, safety shoes, ear protection and safety goggles! - For long hair wear a hairnet or a suitable cap! - Wearing jewelery (rings, chains, watches, visible piercings) is prohibited. - The wearing of gloves is prohibited. - Removing fire loads in the workplace. - Before clamping a grinding wheel Crack test by sound sample. - Clamping a new grinding wheel only by instructed persons. - After clamping test run of 20 minutes. - Watch the tape run and center or tension if necessary. - Check machine for obvious damage. - Workpiece supports and protective covers must be readjusted regularly. - Distance between workpiece support and grinding wheel max. 3 mm. - Attention to follow-up of the machine after switching off! |  |  |  | - pE |
| ## Faults and dangers |  |  |  |  |  |  |
|  |  | - In the event of faults, abnormalities of the machine, breakage and setting of tools, switch off the machine immediately, secure it against commissioning and inform the supervisor. - Only eliminate faults when the machine is at a standstill. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | - Keep Calm - Observe self-protection; Injured people recover, switch off the system - Call first responder and, if necessary, emergency doctor. Emergency call: 144 - Provide first aid - Inform supervisors - An accident report must be made in the case of accidents and near-misses. |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - The machine and the workplace are to be cleaned after use, tools and aids are to be put in the designated places. - Replace damaged grinding wheels and abrasive belts immediately. - Repair and repair only by authorized and authorized persons. - Observe the maintenance schedule of the manufacturer. |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |