|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Handling vacuum, vacuum pumps |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | - Possible implosions due to improper use and / or use of unsuitable material. - Pressure load in a water jet vacuum (20 hPa) approx. 1000 g / cm², in a high vacuum only by 10 - 20 g / cm² more - Risk of injury from splinters |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | General information Do not smoke, eat or drink. Personal protective measures |  |  |  | - pE |
| - Wear safety goggles, face shield and lab coat. - Use only thick-walled, spherical glass containers (no volumetric flasks, stand-up cylinders, Erlenmeyer flasks) - Provide anti-fire protection, wrap either wire basket, acrylic shield, protective wall or glassware with adhesive film / plastic net. - Use boiling boiling capillaries, stones or magnetic stir bars as a result of bumping. - Fasten the devices tension-free (without tilting). - Wear protective goggles with side protection and upper eye area cover. Use: Read the operating instructions of the corresponding device carefully! Before evacuation: Check glass containers for cracks, cracks, "asterisks". Condensate vapors by means of a cold trap or otherwise discharge them safely. For vacuum distillations or rotary evaporators: First evacuate, then heat up Do not heat evacuated equipment judgments on one side. Set to desired speed. The rotation begins |  |  |  |  |  |  |
| ## Faults and dangers |  |  |  |  |  |  |
|  |  | In case of failure immediately remove heat sources, then ventilate the equipment and last stop the pump. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | - Bring injured person out of the danger area - Provide first aid, paying attention to self-protection - Contact the first assistant (see separate notice board), make an emergency call - lnstruct rescue personnel - In the event of injury, only allow glass fragments to be removed by the doctor - Tinnitus :Ears- ringing your ears or ear whistles after an implosion IMMEDIATELY visit doctor!! - Eye contact: In case of eye injury, cover both eyes, ophthalmologist! |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - Do not put laboratory glassware made of Duranglas in the glass container, but dispose of it as residual waste. - The same applies to mirrored glasses (Dewar vessels). |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |