|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work - Equipment - Apparatus -Device |  |  |  |  |  |  |
| Hydrochloric acid smoking 37% |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | Observe information of the safety data sheets Signal word Danger! Hazard statements (H-phrases) H290 can be corrosive against Metal. H314 Causes severe skin burns and eye damage. H335 May cause respiratory irritation Safety instructions (P-phrases) P280 Wear protective gloves / protective clothing / eye protection for prevention. P301 + P330 + P331 IF SWALLOWED: rinse mouth. DO NOT induce vomiting. P305 + P351 + P338 IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if possible. Continue rinsing. P309 + P310 IF EXPOSED OR DISPOSED: Seek medical attention immediately. |  |  |  |  |  |
| ##### Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | General information Eating, drinking or smoking is prohibited in all laboratory facilities! Avoid contact with eyes and skin. |  |  |  | - pE |
| Personal protective measures Eye protection: Tightly fitting safety goggles. Hand protection: nitrile rubber; Thickness: 0.11mm Body protection: Lab coat Respiratory protection: Required if vapors and aerosols are generated. Respiratory mask: Half mask filter class ABEK (as included in emergency kit MUI) Only work in a fume hood! Use and storage Use: Only work in the trigger! Change contaminated clothing immediately Storage: Do not store in metal containers. Keep container tightly closed in a well-ventilated place. Store at a temperature of + 2 ° C to + 25 ° C. Mehr anzeigen Weniger anzeigen Änderung vorschlagenÜbersetzung teilen Transport Transport of the chemical bottle in the safety bottle container. |  |  |  |  |  |  |
| ## Disruptions and Dangers |  |  |  |  |  |  |
|  |  | In case of fire: There are no extinguishing agent restrictions for this substance. Hydrochloric acid is not combustible. After spilling: Do not breathe vapor / aerosol. Avoid substance contact. Ensure adequate ventilation. Soak up with chemical absorbent and have it disposed of by the university's chemical disposal facility. Purify with water. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | First responders: pay attention to self-protection! Symptoms may be delayed. Remove any clothing contaminated with hydrochloric acid. After inhalation: fresh air. Get a doctor. After eye contact: Rinse thoroughly with plenty of water and seek medical advice immediately. After skin contact: Wash off with plenty of water. Remove soiled, soaked clothing immediately. If available, dab with polyethylene glycol 400. See a doctor. Ingestion: Drink water (maximum 2 drinking glasses), avoid vomiting (Perforation). Call a doctor immediately. No neutralization experiments. Most important symptoms and effects, both acute and delayed: Irritation and Irritation and corrosion, coughing, dyspnoea, cardiovascular disorders, Risk of blindness! |  |  |  |  |
| ## Tests - Maintenance - Disposal |  |  |  |  |  |  |
| Recycle empty containers and unusable TRIS by disposing of chemicals at the university. |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |