|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Laminar Flow |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | - Danger due to UV lamp (ozone formation) - danger by gas flame (combustion) |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | - Each use of the laminar flow should be entered in the log book, it should be noted whether with or without Bunsen burner (gas) was worked. - Switch on the UV light for approx. 5 minutes before use. - Do not expose hands and eyes directly to UV light. - Start the laminar flow by o - Opening the sliding glass window (up to the black dot); Acoustic signal sounds - Switch on fan. - Acoustic signal goes out, the laminar flow lights go out, the device is in the warm-up phase (about 3 minutes). - After warming up, the lights come on, the unit is ready for use - If Bunsen burner is used, proceed as follows: - In the control bar, switch on the button with 'socket symbol' (socket), click sound is noticeable. - Turn gas area under tables (2 turns) - Turn right outside of the laminar flow yellow rotary valve (2 turns) - Turn the rotary valve on the bottom left of the Bunsen burner and press on the locking pin, hold it with the lighter during the manual ignition, then release it. - Check that the flame has a bluish color and max. 5-6 cm high. - If necessary regulate via the air supply (rotary knob). - Switching off the Bunsen burner: Close the gas bottle, the flame goes out, close the yellow rotary valve after approx. 10 sec., Then close the rotary valve on the Bunsen burner |  |  |  | - pE |
| ## Faults and dangers |  |  |  |  |  |  |
|  |  | - Close the valve on the gas cylinder immediately in the event of an alarm. - Repair and repair must be carried out exclusively by a specialist workshop. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | - In the event of burns, alert first aiders, rescue service or doctor if necessary. Inform supervisor (if necessary entry in the association book). |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - Maintenance and repair work may only be carried out with the written permission of the laboratory manager. - Only spare parts that have the same material and design as the original parts may be used for maintenance. - Observe the maintenance schedule of the manufacturer. |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |