|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| ### Lathe |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | - Cutting injuries to the tool or chuck. - Detecting clothing and hair through jaw chuck or lead screw. - Danger of crushing by pull-in points. - Injuries due to flying workpiece or tool parts or chips. - Risk of slipping and skin damage due to cooling lubricant. |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | - Operation of the machine only after instruction. - Wear tight-fitting work clothing, safety shoes, and safety goggles! - For long hair wear a hairnet or a suitable cap! - Wearing jewelery (rings, chains, watches, visible piercings) is prohibited - The wearing of gloves is prohibited. - Attention to follow-up of the machine after switching off. - Firmly clamp workpiece and tool. - Remove chuck key before switching on and store securely. - Perform tool and workpiece changes as well as measuring activities only when the machine is at a standstill. - Do not use compressed air for cleaning. - Remove chips only with a hook, broom, brush or industrial vacuum cleaner. NEVER with bare hands. - Modifications to the protective devices are prohibited. |  |  |  | - pE |
| ## Faults and dangers |  |  |  |  |  |  |
|  |  | - In the event of faults, abnormalities of the machine, breakage and setting of tools, switch off the machine immediately, secure it against commissioning and inform the supervisor. - Only remedy faults with the machine switched off and secured. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | - Keep Calm - Observe self-protection; Injured people recover, switch off the system - Call first responder and, if necessary, emergency doctor. Emergency call: 144 - Provide first aid - Inform supervisors - An accident report must be made in the case of accidents and near-misses. |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - The machine and the workplace are to be cleaned after use, tools and aids are to be put in the designated places. - Repair and repair only by authorized and authorized persons. - Observe the maintenance schedule of the manufacturer. |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |