|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Magnetic stirrer with heating plate |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | - The general rules and dangers for handling electrical equipment apply (see relevant operating instructions). - Dangers that are directly caused by the chemicals used are to be looked up in the respective safety data sheets or operating instructions for the chemical and method. - Danger of burning on hot surfaces (hotplate may get over 300 ° C hot). - Caution: Magnetism! Effects of the magnetic field are to be considered (pacemaker). - Risk of electric shock. |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | General information Eating, drinking or smoking is prohibited in all laboratory facilities! Personal protective measures |  |  |  | - pE |
|  Put on protective clothing: lab coat, if necessary gloves and safety goggles  Remove warmed liquids with thermal gloves. Use • Before putting the device into operation, switch off the magnetic stirrer function (set the rotary speed control knob to the left-hand stop), do not start stirring until the stirrer with liquid and magnetic stirrer is already on the stirrer plate. - Keep the mains cable away from the heating plate! - Transport of the device only in the switched off and cooled state. |  |  |  |  |  |  |
| ## Faults and dangers |  |  |  |  |  |  |
|  |  | - In the event of visible damage, do not operate the device. - If faulty, unplug the power cord and inform the responsible manager. - Repair and repair only by a specialist workshop Have customer service carried out. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | First Aider: Pay attention to self - protection! - If necessary, alert first responder, ambulance or doctor. - Still burns for at least 10 minutes (running lukewarm water). If necessary, seek medical treatment. - Inform the supervisor. |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - Maintenance and repair work may only be carried out with the permission of the group leader. - Have repaired and repaired exclusively by a specialist workshop / customer service. - Disposal via old material collection points or specialist dealers. |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |