|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Microwave |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | - The general rules for handling electrical equipment apply (see relevant operating instructions). - Danger due to microwave radiation. - Risk of scalding from hot surfaces or steam. - Risk of scalding due to bumping. - Danger of bursting with closed containers. - Danger due to heating of solvents - Solvents can be dissolved within seconds reach their boiling point! - Fire hazard if excessive heating of solids (e.g., plastics) occurs. |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | General information - Eating, drinking or smoking is prohibited in all laboratory facilities! Personal protective measures |  |  |  | - pE |
| Put on protective clothing: lab coat, disposable gloves, safety goggles Behavior rules - Install microwaves vibration-free. - Only use the microwave oven as intended. - Only operate the microwave oven if the door seal is clean and undamaged - danger the emission of electromagnetic radiation! - Use only suitable laboratory glassware and no closed containers. - Do not introduce metal objects into the microwave - Remove heated vessels from the microwave using a hot-hand or thermo-glove only. - Observe the operating instructions of the manufacturer. - Do not heat food in the microwave. - Do not heat solvents or flammable substances in the microwave. - Do not heat unknown substances in the microwave. - Heat only small quantities in the microwave. - Adjust heating capacity and duration to the substance to be heated - Before using the microwave, inform yourself about it, whether when heating your substance - Dangers can arise that you did not consider before. |  |  |  |  |  |  |
| ## Faults and dangers |  |  |  |  |  |  |
|  |  | - In the event of visible damage, do not operate the device. - Damage to the door is damaged if the door seals are damaged - Electromagnetic radiation can not be ruled out, therefore a microwave oven is allowed not be used with damaged door seals. - In the event of faults, pull out the mains plug and inform the responsible supervisor if possible. - Repair and repair exclusively by a specialist workshop Have customer service carried out. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | First Aider: Pay attention to self-protection! - If necessary, alert first responder, ambulance or doctor. - In case of burns / scalds, cool with water (at least 10 min.). Face and Do not leave the eyes burned. If necessary, arrange for medical treatment. - Inform supervisors. |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - Maintenance and repair work may only be carried out with the permission of the group leader be performed. - Repair and repair only by a specialist workshop / customer service let carry out. - Disposal via waste collection points or specialist dealers. |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |