|  |  |  | Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Milling Machine-Metall |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  |  | - Risk of injury if used improperly (especially for eyes and hands) - Danger of catching by high-speed, rotating machine parts - Risk of injury from flying chips, tools and parts - Risk of skin damage and allergies when handling coolant - Hearing damage due to noise - Danger of electric current, especially in case of damage to electricity of leading line |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  | - Before starting work, check that the machine is in a safe condition - Firmly clamp the workpiece in the vice and remove the wrench - Position protective devices in working position - For tool change, measuring, cleaning etc. Switch off the machine - Wear tight fitting clothes. Jewelry, e.g. Put down rings, chains, bracelets and watches. - Secure longer hair with a hair tie, cap, or headscarf - Wear safety glasses. Ear protection for noisy work - Protective gloves are prohibited: danger of moving into the rotating parts - Do not remove splinters, chips and waste by hand! Switch off the machine and use a chip hook, a hand brush or an approved industrial vacuum cleaner - Wear tight fitting clothes - Wearing gloves and jewelery is prohibited - (Gloves only for material change, with secured, stationary machine) - Use skin protection according to the operating instructions for cooling lubricant - Safety shoes must be worn depending on the size of the workpieces - Observe extended information for different turning techniques. |  |  |  |  | - pE |
| ## Faults and dangers |  |  |  |  |  |  |
|  | - In case of malfunction or damage to machinery or protective equipment - Switch off the machine and secure against unauthorized restarting. - Have damage repaired by qualified personnel only. |  |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  | - Switch off and secure the machine - Inform first responders - Provide immediate injury - Emergency call: 144 |  |  |  |  |  |
| ##### Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - Repair only by authorized and qualified persons - For setup, maintenance and care work, disconnect or secure the machine from the mains - After maintenance, the protective equipment must be checked - Clean the machine after work - Check the coolant regularly according to plan and replace if necessary (see - Operating instructions for cooling lubricants) - E-check every four years by a qualified electrician |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |