|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |  |
| Oven with drying function |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | - There are hazards due to: - In the event of improper operation, safety and security measures may be taken. environmental hazards occur, e.g. if other than the expressly permitted products are filled. - Risk of burns from touching hot surfaces, such as jars, materials, - Risk of fire due to leaked or dropped hot material. - Danger of scalding when extinguishing with water due to explosive formation of water vapor due to the usually very high temperatures. - Danger of crushing fingers in the area of cover plates or, in the case of swiveling ovens, - between oven and rack. |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | Employees are to be made familiar with the operation of the stove, preferably by the manufacturer, before commissioning. - Crucible furnaces may only be operated by technically suitable persons. - Combustible material should be removed from the immediate vicinity of the stove (at least 800 mm). - Never extinguish fires with water or foam. - Only use materials whose properties and the like Melting temperatures are known. - Operation with substances that may release explosive gases when warmed up is prohibited. - Never use the oven without properly installed jars. - Never use the oven to heat food. - No intake of food or drink while working with the oven. - Use suitable heat-resistant PPE to remove materials from the oven (protective gloves, apron, goggles) - Regularly check the safety devices for proper function. - Provide adequate and proper ventilation (away from the person) at the workplace. - Place the stove on a fireproof floor; Furnish the oven environment fireproof. |  |  |  | - pE |
| ## Faults and dangers |  |  |  |  |  |  |
|  |  | • Never use the stove with damaged or missing covers. Inform superiors. - Only carry out troubleshooting by persons who have been trained to do so. are entitled. - Always follow the manufacturer's instructions for troubleshooting. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | - In the event of burns, immediately cool affected areas with sufficiently cold water. - Provide further 1.Aid as far as possible, if necessary visit doctor or call for help. - Instruct emergency personnel and, if necessary, point out special risks. |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| - Cleaning the stove with water or non-flammable cleaning agents. - Before maintenance a. Repair work Disconnect the heater from the power supply and secure against reclosing. - Electrical cables u. Check safety covers regularly for integrity. - Only carry out maintenance work by persons who are suitable for this purpose. - Only carry out work on electrical equipment by a qualified electrician - Work on hydraulic or gas technology. Only equip people with special knowledge and experience in this field. - It is essential to observe the manufacturer's maintenance instructions. |  |  |  |  |  |  |
| ## Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |