|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work - Equipment - Facility – Device |  |  |  |  |  |  |
| Protective Gloves |  |  |  |  |  |  |
| ##### Protection goals |  |  |  |  |  |  |
|  | Moisture-proof gloves soften the skin. This reduces the resistance of the skin to external influences. Gloves may contain allergenic substances. Lengthy skin diseases can be the result. Risk of mechanical injury as a result of skin softening and increased perspiration due to airclosing effect in the glove. Protective gloves: Wearing false gloves causes contamination with chemicals and hazardous agents. |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | General information: Cat. I: Minimal risks, low protection requirement Cat. II: Medium risks, protection against eg. mechanical hazard Cat. III: High risks, protection against irreversible and fatal hazards, e.g. Damage due to chemicals European standards for protective gloves: EN 374 Protective gloves against chemicals and microorganisms EN 388 Protective gloves against mechanical risks EN 407 Protective gloves against chemical risks EN 420 General requirements for gloves EN 421 Gloves for protection against ionizing radiation and radioactive contamination EN 455 Disposable medical gloves EN 511 Protective gloves against cold EN 1082 Protective clothing gloves and armguards to protect against cuts and punctures caused by hand knives EN 1149 Protective clothing against electrostatics EN 12477 Protective gloves for welders Limited chemical protection Chemical risks AKL = letters of the 3 successfully tested chemicals A methanol, B acetone, C acetonitrile, D dichloromethane, E sulphurous hydrocarbon, F toluene, G diethylamine, H tetrahydrofuran, I ethyl acetate, J N-heptane, K sodium hydroxide 40%, L sulfuric acid 96% Only use gloves with CE certificate including four-digit identification number. Observe categories I-III and pictograms. |  |  |  | - pE |
|  |  | Personal protective measures - Read operating instructions - Wash hands with soap and tap water before and after working with chemical protection gloves. - Hands should be clean and dry. - Wear the personal cotton gloves under the chemical protection gloves. - Use hand creams to avoid skin damage. . Use - There is no universal protective glove for all chemical agents. Different gloves must be worn for different chemicals (observe information in safety data sheets). - Gloves used must be suitable for the particular application (for example, observe protective effect and duration of protection). - Before use, check that there are no relevant defects such as cracks, holes, dirt, etc. - Hands should be clean and dry before putting on gloves. - Wrist and hand jewelery must be removed beforehand. - For the change of the gloves hygienic reasons and the duration of the wearing time are to be considered. - Use protective gloves only with clean and dry hands. Fold cuffs to prevent liquids or solutions from leaking into the glove. - Pay attention to maximum load duration! |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
| - At the first signs of damage to the skin (i.e. redness, scaling, itching) as a result of the work, the company doctor or a dermatologist should be consulted and the supervisor informed. - Depending on the hygienic requirements at the workplace, further measures may be necessary (i.e. liquid-tight covering of a small wound). |  |  |  |  |  |  |
| ## Inspections, maintenance, disposal |  |  |  |  |  |  |
| Disposable gloves Disposable gloves may only be used once. Nitrile and Latex Disposable gloves must be disposed of in residual waste. Contaminated nitrile and latex disposable gloves are disposed of in the toxic waste container. Chemical protective gloves If intended to be reused, the gloves should be carefully cleaned before removal and removed so as not to contaminate the inside. Then keep well ventilated with open waistband. Do not lay directly in the sun or on the heater! Store gloves in their original packaging protected from light and temperature! Protective gloves may be disposed of with other toxic contaminated materials. |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |