|  |  |  | ### Operating instructions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ##### Technical Equipment – Devive – Machine - Tool |  |  |  |  |  |  |  |
| Shapers / ram planer |  |  |  |  |  |  |  |
| ##### Environmetal and health Hazards |  |  |  |  |  |  |  |
|  | - Flying chips, moving tools, etc. - Detecting the hair, clothing, jewelery, etc. by means of drive, guide or tool - Cutting, piercing, etc. on tool, workpiece, chips - Intensive skin contact with cutting oil leads to the destruction of the acid mantle, degreasing, dehydration and irritation of the skin as a precursor of skin diseases. - Even minor skin injuries, e.g. due to metal particles, increase the risk of a coolant-related skin disease |  |  |  |  |  |  |
| ## Safety Precautions |  |  |  |  |  |  |  |
|  |  | - Check workstation for defects before starting work. - Take skin protection measures if necessary. - Cover long hair with a cap, hairnet or similar. - Wear tight fitting work clothes with cuffs, roll up sleeves if necessary. - Put down the wristwatch, finger rings, bracelets and loose necklaces, ties, scarves and more. - Gloves must not be worn when bumping. - Firmly clamp the chisel - Wear safety glasses - Do not endanger neighboring workplaces with flying chips or cutting oil (use protective device). - Do not exceed the specified delivery. - Clamp the workpiece firmly in the vice. - To change workpieces, to measure, to clean, etc. Switch off the machine. - Switch off the machine after use and secure it against unauthorized switching on. Remove chips only with a brush or hand brush. Änderung vorschlagenÜbersetzung teilen |  |  |  | - pE |  |
| ## Malfunction and associated Risks |  |  |  |  |  |  |  |
|  |  | - In case of damage or malfunction of the machine: Switch off and inform the instructor. - Remove slipping hazards (eg due to chips or lubricants). - Have the damage repaired by a specialist |  |  |  |  |  |
| ## First Aid |  |  |  |  |  |  |  |
|  |  | - Switch off the machine. - Inform first responders - Take care of injured persons. - Emergency call: 144 |  |  |  |  |  |
| ## Serving – Maintenance - Disposal |  |  |  |  |  |  |  |
| - After completing the work, dispose of chips in the marked collection container. - Clean the machine at the end of work. - Clean up the machine environment at the end of work. - Damage to the machine may only be eliminated by authorized persons. |  |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |  |
| Group leader / Supervisor: Signature: Town: Date: |  |  |  |  |  |  |  |