|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work - Equipment - Apparatus -Device |  |  |  |  |  |  |
| Toluol |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | Signal word Danger! Hazard statements (H-phrases) H225 Highly flammable liquid and vapor. H304 May be fatal if swallowed and enters airways. H315 Causes skin irritation. H336 May cause drowsiness and dizziness. H361d Suspected of damaging the unborn child. H373 May cause damage to organs through prolonged or repeated exposure. Safety instructions (P-phrases) P210 Keep away from heat / sparks / open flames / hot surfaces. Do not smoke. P261 Avoid breathing dust / fume / gas / mist / vapors / spray. P281 Use prescribed personal protective equipment. P301 + P310 IF SWALLOWED: Immediately call a POISON CENTER or doctor / physician. P331 Do NOT induce vomiting. |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | General information Eating, drinking or smoking is prohibited in all laboratory facilities! Avoid contact with eyes and skin. Avoid open flames or sparks! Avoid electrostatic charging! Personal protective measures Eye / face protection Face shield and goggles. skin protection Working with gloves. Gloves must be examined before use. Use a suitable extraction method (without touching the outer glove surface) to avoid skin contact with this product. Material: fluoro rubber Minimum layer thickness: 0.7 mm Breakthrough time: 480 min Protective gloves made of nitrile rubber as short-term splash protection. Layer thickness: 0.4 mm Breakthrough time:> 10 min body protection The type of protective equipment must vary depending on the concentration and amount of dangerous material be selected in the workplace. During laboratory work, always wear a laboratory coat. respiratory protection Work under the deduction !! Do not inhale substance. Avoid development of vapors / aerosols. If, according to the hazard assessment, an air-purifying respiratory protection is required, a half mask filter class ABEK (as included in the MUI emergency kit) must be used in addition to the technical measures. |  |  |  | - pE |
| Use and storage Store in a cool place. Keep container tightly closed in a dry, well-ventilated place store. Close open containers carefully and store upright to prevent leakage. Transfer and transport The transport of liquids in bottles (from about 200 ml) must be done with the aid of a carrying basket. When refilling, pay attention to appropriate protective equipment! Work under the trigger! |  |  |  |  |  |  |
| ## Disruptions and Dangers |  |  |  |  |  |  |
|  |  | Use personal protective equipment. Do not inhale vapors / mist / gas. Ensure adequate ventilation. Remove all sources of ignition. Bring people to safety. Beware of accumulating vapors that can form explosive concentrations. Vapors can accumulate in low-lying areas. In case of fire: Suitable extinguishing agents Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide. Special hazards arising from the substance or mixture Carbon oxides Advice for firefighters In case of fire, wear self-contained breathing apparatus, if necessary. If spilled: Do not inhale vapors. Absorb with liquid-binding material. Dispose of. Clean up affected area. Vapors heavier than air. Environmental precautions: Prevent further leakage or spillage if safe to do so. Do not empty into drains. An entry in the environment is to be avoided. Mehr anzeigen Weniger anzeigen |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | General information Consult a doctor. Show this safety data sheet to the attending physician. After inhalation If inhaled, remove affected person to fresh air. In respiratory arrest, artificial respiration. Consult a doctor. After skin contact Wash with soap and a lot of water. Consult a doctor. After eye contact Rinse thoroughly with plenty of water for at least 15 minutes and consult a doctor. After swallowing DO NOT induce vomiting. Never give anything by mouth to an unconscious person. Rinse mouth with water. Consult a doctor. |  |  |  |  |
| ## Tests - Maintenance - Disposal |  |  |  |  |  |  |
| As a halogen-free, organic solvent waste to the chemical disposal supply. |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |  |