|  |  |  | ### Operating instructions |  |  |
| --- | --- | --- | --- | --- | --- |
| ##### Method |  |  |  |  |  |
| UV-Light |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |
|  |  | UV radiation irritates and damages the eyes (risk of blindness) and may cause skin irritation. High power UV emitters generate ozone when exposed to oxygen. |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |
|  | UV lamps may only be used with the associated ballast (transformer). UV lamps, especially high-power lamps, become very warm and must therefore be operated with effective cooling. Wear UV safety glasses when working with a burning lamp! The goggles must match the power and wavelength of the light source used. Do not look into the burning lamp! Cover exposure equipment, use light-tight sheath (non-flammable), e.g. Aluminum foil. In the case of ozone-developing high-performance lamps, work must be carried out in the fume hood or with an effective source extraction system. |  |  |  | - pE |
| ## Disruptions and dangers |  |  |  |  |  |
|  | Do not use defective UV lamps and avoid further use. Report fault. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |
|  | After eye contact: If the eyes flare, immobilize them with a wide bandage, have the injured transferred to the eye clinic. |  |  |  |  |
| ## Tests – Maintenance - Proposal |  |  |  |  |  |
| Defective mercury lamps are hazardous waste and must be disposed of as mercury waste |  |  |  |  |  |
| ##### Signature |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |