|  |  |  | ### Operating instructions |  |  |
| --- | --- | --- | --- | --- | --- |
| ##### Work equipment – Equipment – Apparatus - Device |  |  |  |  |  |
| Ultrasonic cleaning compact unit |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |
|  |  | - Risk of hearing loss during prolonged operation without hearing protection. - Danger of burning when heating up the liquid. - Danger from shattering glass. |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |
|  | Personal protective measures - Put on protective clothing (at least lab coat, disposable gloves, safety goggles). - In case of continuous activity within 2 m of the ultrasonic cleaner, wear ear protection! Use - Never operate the device without liquid! - Distilled or deionized water without additives is not suitable for ultrasonic cleaning directly in the oscillating tank. - Do not use flammable liquids or chemicals that contain or split off chloride ions for ultrasonic cleaning in the stainless steel pan. - Use aggressive cleaning fluids (such as acids, saline solutions) only in service containers. - Do not put anything on the bottom of the tub! Clean items to be cleaned in the appropriate hanging basket. - Do not put in cleaning liquid during ultrasonic cleaning! - During prolonged operation, the cleaning liquid heats up, therefore, when cleaning. - Temperature-sensitive parts, check the temperature. - When preheating the cleaning fluid at least every 30 min. the liquid stirred or 5 min. Ultrasound be switched on (Bumping!). Business: - Make sure that the process is closed. - Fill the oscillating tank with liquid up to the level mark. - Connect the device to the mains. - Operating Instructions Ultrasonic Bath, Page 2 of 2 - Make sure that the items to be cleaned are completely covered with liquid. - Turn the knob clockwise to the desired cleaning time, the green pilot lamp lights up. Reversing the rotation can shorten the cleaning time or switch off the device. - For continuous operation, turn the knob all the way to the left (∞) and engage. To switch off,turn the knob to the right to "0" |  |  |  | - pE |
| ## Faults and dangers |  |  |  |  |  |
|  | In the event of a fault, switch off the device immediately, disconnect the power plug and inform the supervisor. |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |
|  | - If necessary, alert first responder, ambulance or doctor. superior inform (if necessary entry in the association book). |  |  |  |  |
| - Thoroughly rinse and wipe the stainless steel tray. Do not use steel wool, scratches or scrapers. - Have repairs carried out by authorized specialist personnel only. |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |
| Supervisor: Signature: Town: Date: |  |  |  |  |  |