|  |  |  | ### Operating instructions |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ##### Work - Equipment - Apparatus -Device |  |  |  |  |  |  |
| Xylol, Isomere |  |  |  |  |  |  |
| ##### Dangers for humans and the environment |  |  |  |  |  |  |
|  | Flammable liquid and vapor. (H226) Harmful in contact with skin or if inhaled. (H312 + H332) Causes skin irritation. (H315) Causes serious eye irritation. (H319) Can irritate the respiratory tract. (H335) Can cause damage to organs through prolonged or repeated exposure. (H373) Can be fatal if swallowed and enters airways. (H304) Inhalation, ingestion or absorption through the skin can lead to damage to health. Irritates respiratory tract, eyes, gastrointestinal tract and skin. Ingestion can lead to aspiration in the lungs with subsequent life-threatening pneumonia. Temporary headache, dizziness, nausea, concentration disorders possible. May cause cardiac arrhythmia, kidney damage, nerve damage. Danger due to accumulation of explosive atmosphere near the ground! If ignition sources are present, there is an increased risk of explosion! In the case of rapid leakage from the container, ignition by electrostatics is possible. Increased ignition risk with soaked material (e.g. clothing, cleaning cloth). Reacts with strong oxidizing agents with violent heat development. Reacts under violent heat e.g. with sulfuric acid. In the event of an uncontrolled reaction, there is a risk of explosion. Dangerous reactions at the workplace are possible with: Avoid penetration into soil, water and sewerage! |  |  |  |  |  |
| ## Protective measures and rules of conduct |  |  |  |  |  |  |
|  |  | In case of steam or mist, switch on the suction and work within its working range. Do not open your jaws! When pumping and refilling, avoid spraying and castering. Keep substances capable of reacting at a controlled temperature or only give them in a controlled manner. Workpieces, tools, plant components after cleaning in cleaning devices should preferably be allowed to evaporate in a separate drying area. Thoroughly clean the cleaning device before using a new solvent. Use tools that prevent or reduce skin contact. Keep away from ignition sources (for example not smoke, no open flames, earth)! Only use electrostatically dissipative containers. Connestions to the earth even with tubelines and Armmatures not break. To cover floors only use conductive films, avoid paint residues on the floor. Attach grounding clamps to devices and auxiliary devices. For sampling use a plastic trowel with wooden stick. Limit flow rate during filling. Handle only in dissipative packings. Works with fire only with written permission. Empty the container for cleaning cloths at the workplace every day before work closes. Do not eat, drink, smoke or runny nose. Avoid Inhalation of vapors and aerosols. Contact with eyes, skin and clothing should be avoided! After work and before each pause, hands and other soiled parts of the body should be thoroughly rinsed. Skin care agent is used! Remove product residues immediately from the skin, gently clean the skin and dry thoroughly. Do not wear arm or hand jewelry. Street clothing kept separate from work clothing! Change soiled and saturated work clothes immediately. Use separate cleaning rags and cleaning cloths for skin and machinery or equipment. Observe occupational restrictions! Stock quantity at the workplace: Eye protection: In case of monitoring activity: wearing glasses with side protection! At Danger of splashes: basket glasses! Hand protection: Gloves made of: ... (Please select from the Gis Chem data sheet or other. You will be provided with a suitable protective glove taking into account the activity and will indicate this here.) When wearing protective gloves, cotton underwear gloves are recommended. Pay attention to wearing times of protective gloves! For long-term use of protective gloves: use special skin protection before work. When thinning or filling: Plastic apron! Anti-static protective clothing, e.g. Dress made of cotton! Do not change work clothing in EX areas of Zones 0, 1, 20 or 21, do not wear and do not wear. Wear conductive protective shoes Respiratory mask: Half mask filter class ABEK (as included in emergency kit MUI) |  |  |  | - pE |
| ## Disruptions and Dangers |  |  |  |  |  |  |
|  |  | Clear and close the danger area, inform supervisors. Always wear protective goggles, gloves as well as larger quantities of respiratory protection when cleaning out leaking / spilled product. Respiratory mask: Half mask filter class ABEK (as included in emergency kit MUI). Absorb with absorbent material (e.g., universal binder) and degas! Product is combustible. Insert portable fire extinguisher. Not to be used: water in full jet! In the case of fire, dangerous vapors (for example carbon monoxide) are formed! Strong soot formation! In case of fire in the surrounding area, cool container with spray water! Burst and explosion hazard during heating! Observe alarm, escape and rescue plans. Alert fire department. Occupational Physician: Dr. med. Olaf Riccabona 0512/504 27527, 0664/5210561 Rescue: 144 |  |  |  |  |
| ## Accidents and First Aid |  |  |  |  |  |  |
|  |  | For each first aid measure: pay attention to self-protection, medical treatment. Life-saving immediate measures, such as stable lateral position, cardiopulmonary resuscitation, and shock reduction must be carried out depending on the situation. Cover wounds germ-free. Ensure body peace, protect against heat loss. After eye contact: Immediately wash (with protection of the uninjured eye) extensively (about 10 minutes) with open eyelids with water. After skin contact: Remove uncleaned clothing, also undergarments and shoes, undressed. Rinse skin with plenty of water, if necessary with PEG 400. Inhalation: Move injured person out of the danger zone. Fresh air supply by inhalation of fresh air or respiration. Use ventilation aids (self-protection). Immediately, even in the case of missing signs of illness, inhale a dosing aerosol (inhalable solid). Dosage, method of administration and further treatment according to occupational medical arrangement! Ingestion: Immediate vigorous rinsing of the mouth. Allow water to drip in small swirls. |  |  |  |  |
| ## Tests – Maintenance - Disposal |  |  |  |  |  |  |
| Soak saturated cleaning cloths only in special resistant containers that are tightly sealed. Do not pour into spouts or garbage bins! Collect substance / product waste for disposal in: Packaging with residual contents: Collect uncleaned absorbent material and cleaning cloths in: |  |  |  |  |  |  |
| ##### Signatures |  |  |  |  |  |  |
| Supervisor: SIgnature: Town: Date: |  |  |  |  |  |  |